Example:

Public ip:203.0.113.1(WAN Interface)

Router's internet IP: 192.168.1.1(LAN Interface)

Server's internet IP: 192.168.1.2

Local computer's IP:192.168.1.100

Scenario: the local computer (192.168.1.100) wants to connect to the server(192.168.1.2) using the public IP(203.0.113.1)

How the request is forwarded step-by-step?

Step 1: Local Computer Initiates Connection

The local computer (192.168.1.100) attempts to establish a connection to the destination IP 203.0.113.1 (the public IP of the router).

Source IP: 192.168.1.100

Destination IP: 203.0.113.1

Step 2: Packet Reaches the Router

The packet leaves the local computer, is sent to the default gateway (the router's LAN IP: 192.168.1.1), and is received by the router's internal networking hardware.

Step 3: Router Performs Destination NAT (DNAT) / Port Forwarding Check

The router examines the incoming packet's destination IP address (203.0.113.1). Since this IP address belongs to the router itself (specifically its WAN interface), the router checks its Port Forwarding (NAT) rules.

The router sees the rule: "Traffic arriving on the WAN interface (203.0.113.1:Port) should be forwarded to the internal server (192.168.1.2:Port)."

The router then performs Destination NAT (DNAT):

It changes the packet's Destination IP from 203.0.113.1 to the server's local IP 192.168.1.2.

Packet now looks like this inside the router:

Source IP: 192.168.1.100

Destination IP: 192.168.1.2

Step 4: Router Performs Source NAT (SNAT) / NAT Loopback Handling

Because the packet's original source (192.168.1.100) and new destination (192.168.1.2) are on the same local network segment, the router must perform a second translation step, known as Source NAT (SNAT) or masquerading, to ensure the response traffic knows where to return.

If the router simply forwarded the DNAT-modified packet as is, the server (192.168.1.2) would receive a packet from 192.168.1.100 and try to respond directly to that IP address on the LAN, bypassing the router. The router, which initiated the session mapping, would never see this response packet, and the connection would fail.

To prevent this, the router performs Source NAT:

It changes the packet's Source IP from 192.168.1.100 to its own LAN IP address 192.168.1.1.

It records this session in its NAT table, noting that all return traffic for this specific connection should be un-translated and sent back to 192.168.1.100.

Packet ready for transmission on the LAN looks like this:

Source IP: 192.168.1.1 (Router's LAN IP)

Destination IP: 192.168.1.2 (Server's LAN IP)

Step 5: Packet Delivery to Server

The router transmits this modified packet onto the local network. The server (192.168.1.2) receives a request that appears to originate from the router itself (192.168.1.1).

Step 6: Server Responds

The server processes the request and generates a response packet.

Source IP: 192.168.1.2

Destination IP: 192.168.1.1

Step 7: Response Reaches the Router

The response packet is sent to the default gateway (the router at 192.168.1.1).

Step 8: Router Reverses NAT Translations

The router receives the response and checks its NAT table for the active connection session. It finds the entry it created in Step 4 and performs the reverse translations:

Reverse SNAT: It changes the Destination IP from its own 192.168.1.1 back to the original source, 192.168.1.100.

Response packet looks like this inside the router:

Source IP: 192.168.1.2

Destination IP: 192.168.1.100

Step 9: Final Delivery

The router forwards the finalized response packet to the local computer (192.168.1.100), completing the loop. The local computer successfully receives the data it requested from the "public IP" address.